- Maximum allowable shrinkage strain in RCC per IS 456 (2000) is: 			     	         1

- 0.0030

- 0.0035

- 0.0020

- None of these

- Moment factor, Rlim, for a combination of Fe 415 HYSD steel and M20 concrete according to Limit State design of RCC per IS 456 (2000) is: 							          1

- 2.96 N/mm2

- 2.76 N/cm2

- 2.86 N/cm2

- 2.76 N/mm2

- Minimum residual shear stress in RCC for provision of stirrups, as per IS 456 (2000) is: 											                                 1

- 1.6 N/mm2

- 2.2 N/mm2

- 0.4 N/mm2

- None of these

- What are the minimum and maximum areas of steel in tension in a singly reinforced concrete beam section as per IS 456 (2000)? 						1+1 =2

- A simply supported RC beam has width = 300mm. It is subjected to externally applied load of (10+0.1×last digit of your ID number) kN/m. Assume effective cover = 40mm. Using M25 concrete and Fe415 HYSD steel, find Ast,provided for either singly (change the necessary dimensions accordingly)- or doubly- reinforced section with overall depth limited to 700 mm, as per relevant data from IS 456 (2000) and IS SP-16. Find the corresponding value of shear capacity, τc. Step marks are allocated for units, proper mentions of clause/table/page numbers, relevant sketches, and checks for the necessary allowable limits. Assume any data not provided, with reasonable justifications. 			       25+5 = 30